EXHIBIT “A”

PROPOSED

Revision to

Allegheny County Health Department

Rules and Regulations

Article XXI, Air Pollution Control

§2105.73, “Municipal Solid Waste Landfills”

Revision Tracking No. 99

(Not a SIP Change)

Exhibit A –

Proposed Article XXI Revision –

Municipal Solid Waste Landfills Regulation Revision

Deletions are shown with strikethroughs.

Additions are shown in larger font, bolded, and underlined.

PART E- Source Emission and Operating Standards

….

Subpart 7 – Miscellaneous VOC Sources

….

§2105.73 MUNICIPAL SOLID WASTE LANDFILLS

{Adopted July 15, 1997, Effective August 15, 1997.  Amended mm/dd/2022, effective mm/dd/2022.}

a.	Applicability.  The provisions of this subsection apply to each municipal solid waste landfill that commenced construction, reconstruction or modification on or before May 30, 1991 July 17, 2014 and has accepted waste at any time since November 8, 1987, or has additional design capacity available for future waste deposition.  Physical or operational changes made to an existing municipal solid waste landfill solely to comply with this subsection are not considered construction, reconstruction, or modification and would not subject an existing municipal solid waste landfill to the requirements of New Source Performance Standards for Municipal Solid Waste Landfills.

b.	Each municipal solid waste landfill meeting the conditions of Subsection a shall comply with all of the applicable standards, requirements and provisions of 40 CFR Part 62 Subpart OOO, as amended, which are herein incorporated by reference.

cb.	The provisions of this subsection are in addition to any applicable New Source Performance Standards for Municipal Solid Waste Landfills, or any other Pennsylvania Department of Environmental Protection or Allegheny County permit requirements.

dc.	Definitions of all terms used, but not defined in this subsection, have the meaning given them in 40CFR Part 60 Subpart WWW Part 62 Subpart OOO, as amended.  Terms not defined therein shall have the meaning given to them in the federal Clean Air Act, 40CFR Part 60 Subparts A and B, or this Article.  For the purposes of these rules, “Administrator” shall also mean the Director of the Allegheny County Health Department of or his or her designated representative.

d.	Each municipal solid waste landfill meeting the conditions of paragraph 1 of this subsection shall comply with all of the applicable standards, requirements and provisions of 40CFR Part 60 Subpart WWW, as amended, which are herein incorporated by reference with the exceptions as follows:

1.	Standards for air emissions from municipal solid waste landfills.  The provisions of 40 CFR60.752 are applicable with the exception of the following:

A.	In lieu of 40CFR60.752(a)(2), the following provision applies; When an increase in the maximum design capacity of a landfill exempted from the provisions of 40CFR60.752(b) through 40CFR60.759 on the basis of the design capacity exemption in 40CFR60.752(a) results in a revised maximum design capacity equal to or greater than 2.5 million megagrams and 2.5 million cubic meters, the owner or operator shall comply with the provisions of 40CFR60.752(b) through 40CFR60.759

B.	In lieu of 40CFR 60.752(b)(2)(i)(B), the following provision applies: The collection and control system design plan shall include any alternatives to the operational standards, test methods, procedures, compliance measures, monitoring, recordkeeping or reporting provisions of 40CFR60.753 through 40CFR60.758 proposed by the owner or operator.  In addition, the collection and control design plan must specify: (1) the date by which contracts for control system/process modifications shall be awarded, (which shall be no later than 20 months after the date the NMOC emissions rate is first calculated to meet or exceed 50 megagrams per year); (2) the date by which on-site construction or installation of the air pollution control device(s) or process changes will begin (which shall be no later than 24 months after the date the NMOC emission rate is first calculated to meet or exceed 50 megagrams per year); and (3) the date by which the construction or installation of the air pollution control devices or process changes will be complete.

C.	In lieu of 40CFR60.752(b)(2)(ii), the provisions of paragraphs i and ii below shall apply:

i.	Install and operate a collection and control system capable of meeting the requirements of 40 CFR60.752(b)(2) within 30 months after the date the NMOC emission rate is first calculated to meet or exceed 50 megagrams per year, unless Tier 2 or Tier 3 calculations demonstrate that the NMOC emission rate is less than 50 megagrams per year, as specified in 40CFR 60.757(c)(1) or (2).

ii.	The provisions of 40CFR60.752(b)(2)(ii)(A) and (B) apply as stated therein.

2.	Operational standards for collection and control systems.  The provisions of 40CFR60.753 apply as stated therein.

3.	Test methods and procedures.  The provisions of 40CFR60.754 apply as stated therein with the exception of 40CFR60.754(c), which does not apply.

4.	Compliance provisions.  The provisions of 40CFR60.755 apply as stated therein.

5.	Monitoring of operations.  The provisions of 40CFR60.756 apply as stated therein.

6.	Reporting requirements.  The provisions of 40CFR60.757 apply as stated therein with the exception of the following:

A.	In lieu of 40CFR60.757(a)(1) the following provision applies: The initial design capacity report shall be submitted by the effective date of this Subsection plus 90 days.

B.	In lieu of 40CFR60.757(b)(1)(i), the following provision applies: The initial NMOC emission rate report shall be submitted by the effective date of this Subsection plus 90 days and may be combined with the initial design capacity report required in 40CFR60.757(a).  Subsequent NMOC emission rate reports shall be submitted annually thereafter, except as provided for in 40CFR60.757(b)(1)(ii) and 40CFR60.757(b)(3).

7.	Recordkeeping requirements.  The provisions of 40CFR60.758 apply as stated therein.

8.	Specifications for active collection systems.  The provisions of 40CFR60.759 apply as stated therein.

____________________________________________________________________________________

End of Regulation Changes